NETWORK INFRASTRUCTURE PROJECT

PROJECT TITLE

Group 3

Team Members

| Amgad Mohamed Beshir | 343595 | LinkedIn |
| --- | --- | --- |
| Antonios Sarhan Robel | 338269 | LinkedIn |
| Ayat Abdellateef Attia | 327424 | LinkedIn |
| Sarah Khaled Ibrahim | 322358 | LinkedIn |
| Shahd Ehab | 306722 | LinkedIn |

Supervisor

Dr. Aya Magdy

Assistant Lecturer,

Faculty of Computers and Information

# Table of Contents

1. Project Overview	4

2. Project Objectives	4

3. Team Members & Responsibilities	4

4. Network Topology	5

5. IP Addressing & VLSM Design	8

6. Routing	9

6.1 Routing Design	9

6.2 Configuration	9

6.3 Verification	9

7. Network Security	11

8. VLANs & Inter-VLAN Routing	12

8.1 Design / Configuration	12

8.2 Testing / Verification	12

9. Network Services	13

9.1 Design / Configuration	13

9.2 Testing / Verification	16

10. EtherChannel	16

10.1 Design / Configuration	16

10.2 Testing / Verification	16

11. HSRP	18

11.1 Design / Configuration	18

11.2 Testing / Verification	18

12. WLAN	20

12.1 Design / Configuration	20

12.2 Testing / Verification	20

13. NAT	20

13.1 Design / Configuration	20

13.2 Testing / Verification	21

14. Testing & Verification	21

15. Challenges & Solutions	22

16. Conclusion	23

17. Appendix	23

17.1 GitHub Repository	23

17.2 Project Files	23

# 1. Project Overview

The project implements an enterprise network infrastructure using Cisco Packet Tracer.

The network is divided into a Headquarters (HQ) and a Branch Office. Multilayer switches are used to provide inter-VLAN routing, HSRP is used for gateway redundancy, OSPF provides dynamic routing, EtherChannel provides link aggregation and redundancy, and centralized DHCP, DNS and HTTP services are provided through the Server Farm.

The Edge Router provides NAT/PAT connectivity between the internal enterprise network and the simulated Internet Router.

# 2. Project Objectives

Implement VLSM-based IPv4 addressing.

- Divide the network into multiple VLANs.

- Implement inter-VLAN routing using Multilayer Switches.

- Provide gateway redundancy using HSRP.

- Implement dynamic routing using OSPF.

- Provide centralized DHCP, DNS and HTTP services.

- Implement EtherChannel for link aggregation.

- Provide wireless network connectivity.

- Implement NAT/PAT for Internet connectivity.

- Apply basic network security configurations.

# 3. Team Members & Responsibilities

| Student Name | ID | Primary Responsibilities | Secondary Responsibilities |
| --- | --- | --- | --- |
| Amgad Mohamed Beshir | 343595 | Core network + HSRP | EtherChannel + Redundancy |
| Antonios Sarhan Robel | 338269 | Routing + Edge + Internet | OSBF + NAT + IP Addressing |
| Ayat Abdellateef Attia | 327424 | VLANs + Inter-VLAN | DHCP Relay + Access Switches |
| Sarah Khaled Ibrahim | 322358 | Network Services + WLAN | DHCP + DNS + HTTP + Wireless |
| Shahd Ehab | 306722 | Network Security + Testing | Security verification + Documentation + Presentation |

# 4. Network Topology

# 5. IP Addressing & VLSM Design

| Site | VLAN ID | VLAN Name |  | Subnet Network / CIDR |  | Subnet Mask |  | HSRP VIP (Gateway) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HQ | 10 | HR_Admin |  | 10.10.0.64 /27 |  | 255.255.255.224 |  | 10.10.0.65 |  |
| HQ | 20 | IT_Staff |  | 10.10.0.96 /27 |  | 255.255.255.224 |  | 10.10.0.97 |  |
| HQ | 30 | Wireless_Users |  | 10.10.0.0 /26 |  | 255.255.255.192 |  | 10.10.0.1 |  |
| HQ | 40 | Server_Farm |  | 10.10.0.160 /29 |  | 255.255.255.248 |  | 10.10.0.161 |  |
| HQ | 50 | Voice_VoIP |  | 10.10.0.128 /28 |  | 255.255.255.240 |  | 10.10.0.129 |  |
| HQ | 99 | Management |  | 10.10.0.144 /28 |  | 255.255.255.240 |  | 10.10.0.145 |  |
| Branch | 10 | Branch_Admin |  | 10.10.1.0 /28 |  | 255.255.255.240 |  | 10.10.1.1 |  |
| Branch | 20 | Branch_IT |  | 10.10.1.16 /28 |  | 255.255.255.240 |  | 10.10.1.17 |  |
| Branch | 30 | Branch_Wireless |  | 10.10.1.32 /28 |  | 255.255.255.240 |  | 10.10.1.33 |  |
| Branch | 99 | Branch_Management |  | 10.10.1.48 /28 |  | 255.255.255.240 |  | 10.10.1.49 |  |
| Server Hostname |  |  | Static IP Address |  | Subnet Mask |  | Default Gateway |  | DNS Server |
| Central DHCP Server |  |  | 10.10.0.162 |  | 255.255.255.248 |  | 10.10.0.161 |  | 10.10.0.163 |
| DNS Server |  |  | 10.10.0.163 |  | 255.255.255.248 |  | 10.10.0.161 |  | 10.10.0.163 |
| Web Server |  |  | 10.10.0.164 |  | 255.255.255.248 |  | 10.10.0.161 |  | 10.10.0.163 |

| Link Connection Target | Device A | Interface A | IP Address A | Device B | Interface B | IP Address B | Subnet / Mask |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HQ to Branch Mesh Link 1 | HQ-MLS1 | Gi1/0/7 | 10.10.2.1 | Branch-MLS1 | Gi1/0/1 | 10.10.2.2 | 10.10.2.0 /30 |
| HQ to Branch Mesh Link 2 | HQ-MLS1 | Gi1/0/8 | 10.10.2.5 | Branch-MLS2 | Gi1/0/1 | 10.10.2.6 | 10.10.2.4 /30 |
| HQ to Branch Mesh Link 3 | HQ-MLS2 | Gi1/0/7 | 10.10.2.9 | Branch-MLS1 | Gi1/0/2 | 10.10.2.10 | 10.10.2.8 /30 |
| HQ to Branch Mesh Link 4 | HQ-MLS2 | Gi1/0/8 | 10.10.2.13 | Branch-MLS2 | Gi1/0/2 | 10.10.2.14 | 10.10.2.12 /30 |
| HQ Core to Edge Uplink 1 | HQ-MLS1 | Gi1/0/9 | 10.10.2.25 | EDGE-R | Gi0/0/0 | 10.10.2.26 | 10.10.2.24 /30 |
| HQ Core to Edge Uplink 2 | HQ-MLS2 | Gi1/0/9 | 10.10.2.29 | EDGE-R | Gi0/0/1 | 10.10.2.30 | 10.10.2.28 /30 |
| Edge Router to ISP Link | EDGE-R | Gi0/0/2 | 10.10.2.33 | INTERNET-R | Gi0/0/0 | 10.10.2.34 | 10.10.2.32 /30 |
| ISP Internet Host | INTERNET-R | Loopback 0 | 8.8.8.8 | N/A | N/A | N/A | 8.8.8.8 /32 |

# 6. Routing

## 6.1 Routing Design

We used OSPF (Open Shortest Path First) in Area 0 for dynamic internal routing. OSPF allows all Multilayer Switches and Routers to dynamically discover subnets and automatically reroute traffic if a physical link fails.

Additionally, a Static Default Route (0.0.0.0 0.0.0.0) is configured on the Edge Router (EDGE-R) pointing toward the ISP. This default route is redistributed into OSPF so all internal switches can reach external Internet destinations.

## 6.2 Configuration

1. OSPF Configuration on Core Switch (HQ-MLS1):

HQ-MLS1(config)# router ospf 1

HQ-MLS1(config-router)# router-id 1.1.1.1

HQ-MLS1(config-router)# network 10.10.0.0 0.0.255.255 area 0

2. Default Route & OSPF Redistribution on Edge Router (EDGE-R):

EDGE-R(config)# ip route 0.0.0.0 0.0.0.0 10.10.2.34

EDGE-R(config)# router ospf 1

EDGE-R(config-router)# router-id 5.5.5.5

EDGE-R(config-router)# network 10.10.2.24 0.0.0.3 area 0

EDGE-R(config-router)# network 10.10.2.28 0.0.0.3 area 0

EDGE-R(config-router)# default-information originate

## 6.3 Verification

Routing operation and neighbor adjacencies were verified using the following Cisco IOS verification commands:

- "show ip ospf neighbor": Confirms that all Core switches and Routers established full OSPF neighbor relationships.

- "show ip route": Confirms that all internal subnets and the OSPF default route (O*E2 0.0.0.0/0) are properly populated in the routing table.

- "ping / traceroute": Verified 100% end-to-end connectivity between internal VLAN PCs and external ISP IP address.

# 7. Network Security

| Security Feature | Device(s) | Purpose | Verification |
| --- | --- | --- | --- |
| PAT (NAT Overload) | EDGE-R Router | Hides internal IP addresses behind a single Public IP for internet security. | Show ip nat translations |
| VLAN Segmentation | Core & Access Switches | Isolates department traffic to prevent unauthorized network access. | show vlan brief |
| Secure SSH Protocol | All Routers & Switches | Encrypts remote management sessions instead of clear-text Telnet. | Show ip ssh |
| STP PortFast & BPDU Guard | Access Switches | Prevents unauthorized switches from connecting and disrupting topology. | show spanning-tree summary |
| Voice VLAN Separation | Access Switches | Protects VoIP phone calls from data network sniffing. | show interface switchport |

# 8. VLANs & Inter-VLAN Routing

## 8.1 Design / Configuration

We created 5 distinct VLANs to separate network traffic:

- VLAN 10 (HR_Admin)

- VLAN 20 (IT_Staff)

- VLAN 30 (Wireless_Users)

- VLAN 40 (Server_Farm)

- VLAN 50 (Voice_VoIP).

Inter-VLAN Routing is configured on Multilayer Switches (HQ-MLS1 and HQ-MLS2) using Switched Virtual Interfaces (SVIs) for fast hardware-based routing between subnets.

Configuration Example (HQ-MLS1):

HQ-MLS1(config)# vlan 10

HQ-MLS1(config-vlan)# name HR_Admin

HQ-MLS1(config-vlan)# interface vlan 10

HQ-MLS1(config-if)# ip address 10.10.0.66 255.255.255.224

HQ-MLS1(config-if)# ip helper-address 10.10.0.162

HQ-MLS1(config)# ip routing

## 8.2 Testing / Verification

Inter-VLAN Routing and Trunk configurations were verified using the following steps:

1. Executed "show vlan brief" to verify VLAN creation and access port assignments.

2. Executed "show interfaces trunk" to ensure 802.1Q encapsulation is active on all switch-to-switch links.

3. Performed Ping tests between hosts in different VLANs (e.g., HR PC in VLAN 10 pinging IT PC in VLAN 20), achieving 100% success rate.

# 9. Network Services

## 9.1 Design / Configuration

We centralized core network services inside the Server Farm (VLAN 40) using dedicated servers:

- DHCP Server (IP: 10.10.0.162):

Configured with IP scopes for each VLAN. The command "ip helper-address 10.10.0.162" is added on SVI interfaces so clients receive dynamic IPs across different VLANs.

- DNS Server (IP: 10.10.0.163):

Resolves domain names (e.g. enterprise. local) to server IP addresses.

- HTTP Web Server (IP: 10.10.0.164):

Hosts the corporate internal web portal.

SVI Configuration Example (DHCP Relay):

HQ-MLS1(config)# interface vlan 10

HQ-MLS1(config-if)# ip helper-address 10.10.0.162

## 9.2 Testing / Verification

1. DHCP Verification: Connected PC clients in VLAN 10 and VLAN 20, set their IP configuration to "DHCP", and verified they automatically received correct IPs, Default Gateways, and DNS addresses.

2. Web/DNS Verification: Opened web browser on user PCs and navigated to "enterprise.local", confirming successful HTTP connection to the web server.

# 10. EtherChannel

## 10.1 Design / Configuration

We configured LACP (Link Aggregation Control Protocol - IEEE 802.3ad) to bundle physical links between switches into logical Port-Channels. This doubles bandwidth and provides link redundancy.

- Port-Channel 1 (Between HQ-MLS1 & HQ-MLS2):

Bundles two 1 Gbps links (Gig1/0/4 and Gig1/0/5) into a 2 Gbps link.

- Port-Channel 2 (Between Access Switches):

Bundles FastEthernet links using LACP "active" mode.

Configuration Example (HQ-MLS1):

HQ-MLS1(config)# interface range GigabitEthernet1/2/1 - 2

HQ-MLS1(config-if-range)# channel-group 1 mode active

HQ-MLS1(config-if-range)# exit

HQ-MLS1(config)# interface port-channel 1

HQ-MLS1(config-if)# switchport mode trunk

## 10.2 Testing / Verification

1. Executed "show etherchannel summary" to confirm Port-Channel status shows "SU" (In Use / Layer 2 Trunk) and member ports show "P" (Bundled in Port-Channel).

2. Physically disconnected one member link during ping testing; traffic continued seamlessly over the second link without packet loss.

# 11. HSRP

## 11.1 Design / Configuration

We implemented Hot Standby Router Protocol (HSRP) between HQ-MLS1 and HQ-MLS2 to provide First-Hop Redundancy. This creates a shared Virtual Gateway IP address so users never lose internet or network access if one Core Switch fails.

- Virtual IP (VIP):

Configured per VLAN (e.g., 10.10.0.65 for VLAN 10).

- HQ-MLS1 (Active):

Priority set to 110 with "preempt" enabled to always serve as the primary gateway when online.

- HQ-MLS2 (Standby):

Priority default set to 100 as the backup gateway.

Configuration Example (VLAN 10 on HQ-MLS1):

HQ-MLS1(config)# interface vlan 10

HQ-MLS1(config-if)# standby 10 ip 10.10.0.65

HQ-MLS1(config-if)# standby 10 priority 110

HQ-MLS1(config-if)# standby 10 preempt

## 11.2 Testing / Verification

1. Executed "show standby brief" to verify HQ-MLS1 status is "Active" and HQ-MLS2 status is "Standby".

2. Initiated a continuous ping from a VLAN 10 host to an external IP, then manually shut down the active interface on HQ-MLS1.

3. HQ-MLS2 instantly transitioned to "Active" state with sub-second failover, allowing the continuous ping to succeed with zero downtime.

# 12. WLAN

## 12.1 Design / Configuration

We extended wireless connectivity to mobile and guest users inside VLAN 30 using Wireless Access Points (APs).

- SSID: Enterprise_WiFi

- Subnet / VLAN: VLAN 30 (10.10.0.0/26)

- Security: WPA2-Personal (AES Encryption) for secure wireless authentication.

- IP Assignment: Wireless clients dynamically receive IP parameters from the central DHCP Server via VLAN 30 SVI.

## 12.2 Testing / Verification

1. Connected wireless laptop clients to "Enterprise_WiFi" SSID using the pre-shared key.

2. Verified that wireless clients successfully obtained IP addresses in the 10.10.0.1 - 10.10.0.62 range.

3. Executed Ping tests from wireless clients to internal servers (VLAN 40) and external destinations, confirming full connectivity.

# 13. NAT

## 13.1 Design / Configuration

We configured Port Address Translation (PAT / NAT Overload) on the Edge Router (EDGE-R). PAT translates all internal private IP addresses (10.10.0.0/16) into a single public IP address when connecting to the Internet, providing IPv4 address conservation and perimeter security.

- Inside Interface: GigabitEthernet0/0/0 (Facing internal LAN)

- Outside Interface: GigabitEthernet0/0/1 (Facing ISP)

Configuration Example (EDGE-R):

EDGE-R(config)# access-list 1 permit 10.10.0.0 0.0.255.255

EDGE-R(config)# ip nat inside source list 1 interface GigabitEthernet0/0/1 overload

EDGE-R(config)# interface GigabitEthernet0/0/0

EDGE-R(config-if)# ip nat inside

EDGE-R(config)# interface GigabitEthernet0/0/2

EDGE-R(config-if)# ip nat outside

## 13.2 Testing / Verification

1. Initiated HTTP traffic and Ping requests from an internal PC in VLAN 10 toward the external ISP IP address.

2. Executed "show ip nat translations" on EDGE-R to confirm active translations, showing internal private IPs mapped to the router's outside interface port numbers.

# 14. Testing & Verification

| Test ID | Feature | Test Procedure | Expected Result | Actual Result |
| --- | --- | --- | --- | --- |
| T01 | End-to-End Connectivity | Ping from VLAN 10 PC to ISP External IP | 100% success rate with active reply | PASS |
| T02 | Routing | Verify routing table using show ip route | OSPF routes and 0.0.0.0/0 default route visible | PASS |
| T03 | DHCP / DNS / HTTP | Request dynamic IP on client PC and open web portal | IP assigned automatically & web page loads | PASS |
| T04 | VLAN / Inter-VLAN | Ping between PC in VLAN 10 and PC in VLAN 20 | Successful inter-subnet communication | PASS |
| T05 | EtherChannel | Disconnect 1 physical link in Port-Channel 1 | Traffic continues seamlessly over 2nd link | PASS |
| T06 | HSRP Failover | Shutdown active interface on HQ-MLS1 switch | Standby HQ-MLS2 instantly becomes Active | PASS |
| T07 | WLAN | Connect Laptop via Wi-Fi SSID with WPA2 | Wireless client gets IP and reaches network | PASS |
| T08 | NAT | Check translations using show ip nat translations | Internal IPs translated to public interface IP | PASS |
| T09 | Security | Attempt Telnet connection to Core Switch | Connection refused; only SSH allowed | PASS |

# 15. Challenges & Solutions

| Challenge | Root Cause | Solution |
| --- | --- | --- |
| Inter-VLAN DHCP Request Failure | DHCP Relay wasn't forwarding broadcast packets across routers/switches. | Added ip helper-address 10.10.0.162 on all VLAN SVI interfaces. |
| EtherChannel Mismatch / Discarding | Member ports had inconsistent speed, duplex, or trunking settings. | Standardized port settings before applying channel-group 1 mode active. |
| HSRP Preemption Flapping | Both Core Switches claimed Active role simultaneously during boot up. | Configured HSRP preempt delay and fine-tuned priority values on HQ-MLS1. |
| NAT Translation Loss for Wireless Users | Access Control List (ACL) did not include the new WLAN | Updated NAT ACL to permit the whole private address space 10.10.0.0 0.0.255.255. |

# 16. Conclusion

The Enterprise Network Infrastructure project was successfully designed, configured, and tested. By adopting a hierarchical network design combined with VLSM IPv4 addressing, the architecture achieves high scalable capacity, fault tolerance, and security. Key redundancy protocols such as HSRP and EtherChannel ensure zero downtime for critical business operations, while OSPF dynamically manages core routing. The integration of centralized network services (DHCP/DNS/HTTP), secure WLAN access, and PAT for internet traffic complete a robust, reliable, and production-ready enterprise network solution.

# 17. Appendix

## 17.1 GitHub Repository

GitHub Link: https://github.com/amgadnassar74/Group3_NTI_project_Network

## 17.2 Project Files

Google drive link (including packet tracer file, excel sheet, documentation and screenshots):

Project (1) - Google Drive